Mattin Mir-Tahmasebi (mm5213) and Ahmed Ibrahim (aai13)

March 2016

RTDSP Lab 5 Report

Real-Time Implementation of IIR Filters

Imperial College London

Contents

Single pole RC filter	2

Direct I Form IIR filter Implementation in C	4

RC Filter Analysis	5

Comparison of Digital and Analogue response	8

Bandpass Filter: Direct Form II	12

Direct Form II Analysis	14

Bandpass filter: Direct Form II Transposed	19

Performance Comparison	22

# Single pole RC filter

Initially, a single pole filter, consisting of a simple RC filter, must be designed. This will be a first-order low pass filter. The mathematical representation of the filter in the s domain, shown below, will be mapped in the discrete time domain using the Tustin (bilinear) transform.

Figure 1 Simple low pass RC filter

The transfer function for this filter can be found to be:

VoutVin=1sC1sC+R

∴Hs=1sC1+sRCsC

Hs=1sRC+1

Using the Tustin transform we can map this function to the discrete time domain by substituting:

s=2T1-z-11+z-1

The transfer function now becomes:

Hz=12T1-z-11+z-1RC+1

Hz=12T1-z-1RC+11+z-1

Therefore:

Hz=1+z-12TRC+1+1-2TRCz-1

Using the values given in the specification (R = 1KΩ, C = 1µF) and the sampling frequency (8000Hz), the transform becomes.

Hz=1+z-117-15z-1

To implement this filter in C, the characteristic transfer function of an IIR filter must be used to give a difference equation in the time domain.

Hz=b0+b1z-1+b2z-2+…+bMz-M1+a1z-1+a2z-2+…+bNz-N

In Direct form I:

Figure 2 Direct-Form I

A corresponding difference equation can be found:

yn=b0xn+b1xn-1+…+bMxn-M-a1yn-1-a2yn-2-aNyn-N

Manipulating the discrete time transfer function to have it in the form of an IIR filter:

Hz=1+z-1∙117 1-1517z-1

The corresponding difference equation is:

yn=117xn+117xn-1--1517y(n-1)

Therefore, b0=b1=117 , a1=-1517 .

## Direct Form I IIR filter Implementation in C

Using the coefficient values calculated above, the RC filter was implemented in C.

It makes use of two buffers, xt[] and yt[] (Figure 3) which hold the history of input and outputs, respectively, where the 0th index represents the newest sample, and increasing indices indicate samples from further back in time.

Figure 3 Global variable initialization. xt and yt for the RC filter and x  for direct form 2 filters

The algorithm begins by shifting the past inputs back in the buffer to make room in xt[0], which is initialised to the incoming sample.  The filter is then applied using the coefficients calculated earlier and assigned to the start of the output buffer. That value is also assigned to the next element in the buffer for use in the next iteration of the filter, and is finally assigned to the output. The filter function is seen in Figure 4.

Figure 4 RC filter C implementation

## RC Filter Analysis

The filter is then implemented in the DSK. Driving the input with a low frequency square wave, as in Figure 5, the time constant of this RC filter can be found.

Figure 5 RC filter output for a square wave input

An input frequency of 150Hz is used for this filter since it is well below the cut off frequency for the first order filter (Figure 6) (fcut-off=12πRC≈160Hz) and is also above the roughly 7Hz cut-off of the high pass filter present at the output of the AIC23 Codec.

Figure 6 RC filter ideal response showing the roughly 160Hz cut-off frequency

The time constant of the filter can be found by driving the input with this 150Hz square wave. Using the output given in Figure 7, the time constant for the RC filter can be found by using the relationship Vc=Vsup1-1e-tτ  where τ = RC in this case.

Figure 7 150Hz square wave input with its corresponding output

By calculating the time for the output voltage to decay by one time constant, which is when the output voltage decreases to 36.8% of its initial peak value, the time constant of this RC IIR filter can be found. An example is shown on Figure 8 below.

Figure 8 Capacitor discharging voltage example

Using the output trace, the time constant can be found (Figure 9).

Figure 9 Finding the time constant using the output voltage trace

The value of the time constant is found to be 0.8ms. This is below the expected value of 1ms (τ=RC=1ms). This can be explained due to the effects of high pass and reconstruction filters present in the DSK (see next subsection). The DSK attenuates the response expected from the ideal RC filter and gives a response as seen in Figure 10.

Figure 10 Response of the RC filter as given by the spectrum analyser

The time constant can be found for the ideal filter by subtracting the non-filtered response from the response of the ideal filter, this is given in Figure 6.

Figure 11 Adjusted response for the RC filter taking into account the attenuation of the DSK

Finding the cut-off frequency for this gives a time constant of 0.99 which is very close to the expected value of 1ms.

## Comparison of Digital and Analogue response

The ideal response of the filter is shown in Figure 12. The actual response of the filter as given by the spectrum analyser is shown in Figure 10. It can be seen that the response does not closely match the expected magnitude response given by MATLAB in Figure 6. This can be explained by looking at the spectrum analyser magnitude response when no filter function is used. The response for this is shown in Figure 13 below.

Figure 12 Bode plot of ideal RC filter

Figure 13 Spectrum analyser magnitude response when no filter function is used

As can be seen there is attenuation at low frequencies (~<10Hz) which is due to the high pass filter present at the output of the AIC23 Codec (Figure 14). The transfer function for this filter is s=sCR2sCR1+R2+1=0.02209s0.02214s+1  . The magnitude response of this filter can also be plotted and is shown in Figure 15. With a cut-off frequency of 7Hz, this shows the attenuation of the RC filter at low frequencies. At high frequencies there is sharp drop due to the reconstruction filter with a cut-off frequency of fs2. This is also shown clearly on the actual RC filter plot (Figure 10). The -12dB attenuation is due to the input to the AIC23 Codec being halved due to the potential divider and the fact that the two inputs to the spectrum analyser are being averaged: since only one line is used the input amplitude is halved again.

Figure 14  AIC 23 Codec Audio Chip

Figure 15 Bode plot for the high pass filter at the output of the AIC23 Codec

Superimposing the spectrum analyser results with the ideal analogue filter results, the attenuation from the reconstruction and high pass filters are seen clearly (Figure 16).  The attenuation brought about by the hardware is also seen. The reconstruction filter attenuation at 4Khz can be seen when plotting the digital filter. The spectrum analyser results can now be superimposed onto the ideal digital filter response. This is shown clearly in Figure 17.

Figure 16 Actual spectrum results superimposed onto the ideal magnitude response

Figure 17 Actual spectrum superimposed onto the ideal digital response

# Bandpass Filter: Direct Form II

An elliptic bandpass filter is to be designed with the following specifications:

| Order | 4th |
| --- | --- |
| Passband | 180-450Hz |
| Passband Ripple | 0.4dB |
| Stopband Attenuation | 23dB |

Table 1 Filter Specifications

An elliptic filter has ripple in both pass-band and stop-band. This allows the filter to have an extremely fast transition (faster than a Chebyshev) between the passband and stopband. An elliptic filter has the following response:

Hjω2=11+ ε2Rn2(ω,L)

Where Rnω,L is the response of a Chebyshev filter, and ε is the ripple factor. Elliptic filters are generally used where it is especially important that cut-offs be at particular frequencies.

Assuming a sampling frequency of 8Khz, the elliptic filter can be designed in MATLAB using the program shown in Figure 18. The MATLAB program returns a set of filter coefficients that can be used to implement the IIR filter in Direct form II. The response of the filter is shown in Figure 19. It can be seen from the response that the filter has a sharp roll-off at the cut-off frequency.

Figure 18 The design of the filter in MATLAB

Figure 19 Filter frequency response

To implement the Direct Form 2 filter, we consider each adder shown in Figure 21 as two separate variables, left and right.

For every new sample that is received, the existing samples in the buffer are shifted down, and the relevant filter coefficients are applied, the results of which are added to the respective variable: left for the a coefficients, and right for the b coefficients.

The new sample is then added to the left adder, which is multiplied by b[0] and added to right to create the final value of the output. Lastly, the current value of left is assigned to the first element of the buffer, to be ready for the next cycle. The C function used is shown in Figure 20.

Figure 20 Direct form II C implementation

Figure 21 Direct Form II

## Direct Form II Analysis

The analyser spectrum given in Figure 22 shows that the magnitude response is similar to the response given by MATLAB. The pass-band ripple, as seen in Figure 23, is measured to be 0.390dB, well within the specified limits. The actual spectrum can be superimposed onto the spectrum given by MATLAB (Figure 24). It can be seen that the actual spectrum clearly matches the ideal magnitude response, however at low frequencies, there is attenuation. The plot also is taken at a relative level, removing the 12dB attenuation performed by the hardware from the plot. The low frequency attenuation, as explained earlier, is due to the high pass filter present at the output of the AIC23 codec. The high frequency attenuation, also mentioned earlier, is due to the reconstruction anti-aliasing filter present in the DSK. The phase response shown in Figure 25 is, however, quite different. There is an additional phase shift, again due to the hardware. Again the AIC23 codec gives this additional shift, which can be seen when looking at the phase response using the spectrum analyser without a filter implemented (Figure 26).

Figure 22 Magnitude response (250Hz relative level)

Figure 23 Spectrum analyser response showing pass-band ripple within the specified limits

Figure 24 Actual spectrum superimposed onto the ideal magnitude response. Note that the actual spectrum is used at a relative level.

Figure 25 Actual phase response of the IIR filter showing an additional phase shift

Figure 26 Phase response when no filter function is applied

Another interesting effect can be seen when increasing the order of the filter. As the filter order is increased past 12, the response given by MATLAB shows significant distortion to the required magnitude and phase response. This distorted frequency response is shown in Figure 27. The distortion is due to the filter becoming unstable at higher orders. This can only be explained by looking at the placement of the poles and zeros of the filter.

Figure 27 Distorted frequency response of 14th order elliptic filter

When using a 4th order filter the poles of the filter are placed well within the unit circle allowing for inherent stability. The 4th order filter, computed by MATLAB, is given by the transfer function:

Hz=0.0759-0.2346z-1+0.3175z-2-0.2346z-3+0.0759z-4 1-3.5748z-1+4.8975z-2-3.0531z-3+0.7325z-4

It has the z-plane plot as shown in Figure 28 below:

Figure 28 z-plane plot of a 4th order elliptic filter

The plot shows that the poles are within the unit circle. When using a higher order filter however, to achieve the steep roll-off, the poles are moved closer to the unit circle. An example is shown with an 8th order elliptic filter on Figure 29 below.

Figure 29 Z plane plot for an 8th order elliptic filer

This causes problems as the filter order increases. When reaching a filter of order 14 the poles are placed outside the unit circle causing instability. This is due to the double-precision floating point data type used by MATLAB, whose finite precision means it can be subject rounding and quantisation errors, leading to incorrect calculation of the position of the poles of the filter to be outside of the unit circle. This is shown in Figure 30 when using a 14th order filter.

Figure 30 z-plane plot of a 14th order filter showing the poles are placed outside the unit circle

To determine the linear equation that describes the relationship between filter order and number of cycles taken for each algorithm, the algorithms were run at several different order levels, and the time taken, in cycles, was recorded. This process was repeated for no optimisation and optimisation level 2 for two algorithms (see next section for the second algorithm). The results were collated in Table 2, and a graph was plotted from these results (Figure 31), showing a linear relationship. When no optimization is used, the number of instruction cycles of a filter of order n has the linear relationship 76n + 30. When using optimisation 2, the relationship is of the form 6.2n +109.2.

| Order | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- |
| No optimisation | 334 | 486 | 638 | 790 | 942 |
| Optimisation 2 | 136 | 145 | 157 | 171 | 185 |

Table 2 Performance (in cycles) for a Direct Form II IIR filter for different filter orders

Figure 31 Graph of number of cycles vs filter order for a direct form II IIR filter

# Bandpass filter: Direct Form II Transposed

The IIR filter is now implemented in the Direct Form II transposed structure. This form works by summing the chained outputs of previous adders. The output is initialised to be the sum of the first output and the first b coefficient applied to the new sample, as shown in Figure 33. The for loop in the function sets the value of the output of each adder (represented by x[i]) as the sum of the output of the previous adder, and the ith a and b coefficients applied to the new sample. The reason it only runs until BUFSIZE-2 is that the last element, i.e. the last adder (position BUFSIZE-1 in the buffer), has no adder previous to it, so its output is calculated slightly differently:  the last b coefficient applied to the new sample, minus the last a coefficient applied to the output. Figure 32 shows the function implemented in C.

Figure 32 Direct form II transposed function

Figure 33 Direct form II transposed form structure of the IIR filter

When the C code is implemented, the response of the filter (Figure 34), closely matches that of the previous non transposed Direct Form II IIR filter. Similar characteristics are also seen due to the hardware attenuation. As can be seen from Figure 34, again there is attenuation at low frequencies (~7Hz) and at close to the Nyquist frequency. The ripple is calculated and is found to be well within the specification (0.391dB, shown in Figure 35). The convolution summation implemented is now:

yn= k=1Mbkxn-k-akyn-k+b0x(0)

Figure 34 Response of Direct form II transposed

Figure 35 Pass-band ripple, within specification

Using the data collected in Table 3, the graph in Figure 36 was produced, showing that there is, indeed, a linear relationship. This is especially true for the non-optimised processes, where the number of cycles increased by exactly the same amount for each data point (152 for Direct Form 2 (Figure 31) and 118 cycles for Direct Form 2 Transposed).  The optimised processes' first two data points are slightly less linear, but from the this data point onward, they appear to settle into a linear trend, growing at the same number of cycles (14 cycles for Direct Form 2 and 10 cycles for Direct Form 2 Transposed).

For the Direct Form 2 Transposed, the relationship between order and number of cycles is 59n + 38 for a non-optimised function. When using optimisation level 2, the relationship between order and number of cycles is 5.2n + 107.8.

| Order | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- |
| No optimisation | 274 | 392 | 510 | 628 | 746 |
| Optimisation 2 | 129 | 138 | 160 | 170 | 180 |

Table 3 Results for the Direct Form II transposed IIR filter implementation

Figure 36 Direct Form II transposed graph of number of cycles vs filter order showing a linear trend

# Performance Comparison

Using the previous results and algorithms, the performance of the two filters can be compared. The transposed form is seen to be faster than the non-transposed form. Looking at the non-optimised plots, the transposed form has a much lower n coefficient (59) than the non-transposed form (76). This means that it will stay consistently faster as order is increased. This is due to the differences in their for loops: in the non-transposed algorithm, we are performing three distinct operations (two multiplications and a shifting of memory along the buffer), whereas in the transposed version, we are performing similar operations, but have grouped them into one statement. Since the instructions are not being optimised, the compiler is less efficient at managing its memory with the non-transposed algorithm, since it is not neccessarily aware that the values it is calculating will be summed at the end. In the transposed algorithm, however, this is more clearly communicated to the compiler.

Looking at the equations for the -o2 plots, there is slightly more overhead in the Direct Form II algorithm than the transposed version (109.2 vs 107.8). Looking at the C code, the non-transposed algorithm has more initialisations, which is likely what is contributing to that increase, however, it is difficult to accurately predict why the optimised code behaves as it does, since the compiler alters the code in non-obvious and sometimes counter-intuitive ways to maximise speed. Looking at the disassembly (for a 12th order filter) in Figure 37, we find that the transposed form makes use of 42 parallel-execution instructions in its for loop, as opposed to the non-transposed form's 28, which may be a contributing factor, as it reduces the number of cycles needed to run one iteration of the loop.

Figure 37 Example of the parallel-execution instructions, preceded by ||

https://upload.wikimedia.org/wikipedia/commons/thumb/e/e0/1st_Order_Lowpass_Filter_RC.svg/2000px-1st_Order_Lowpass_Filter_RC.svg.png

Real Time Digital Signal Processing Lab 5 Coursework notes – Implementation of IIR Filters page 5

http://www.electronics-tutorials.ws/rc/rc5.gif?81223b

Real Time Digital Signal Processing Lab 5 Coursework notes – Implementation of IIR Filters page 6

http://www.radio-electronics.com/info/rf-technology-design/rf-filters/elliptic-cauer-rf-filter-basics.php

http://www.fizyka.umk.pl/~daras/pfc/filtr_eliptyczny.pdf

Real Time Digital Signal Processing Section 6 - Filters and their Design P11